Spectral Flux Density Coverage

To create a scenario that shows Spectral Flux Density, follow these steps:

- Create a satellite object with a transmitter and a facility object with a receiver.

- Make sure the transmitter is set to Data Rate = 2 kbps. This will ensure a 4 kHz Bandwidth using BPSK modulation.

- Create an access between the transmitter and receiver.

- Open the Analysis Workbench.

- Select the Calculation tab and then the Access object.

- Create a Scalar Calculation of Data Element type called FD (Flux Density).

- Select Flux Density and clear the Use Samples checkbox.

- Click OK.

- Create a Scalar Calculation of Data Element type called BW (BandWidth).

- Select Bandwidth and clear the Use Samples checkbox.

- Click OK.

- Select the Receiver on the left.

- Create a Scalar Calculation of Function(x,y) type called SFD (Spectral Flux Density).

- Make sure the Function = a*x/y (division function).

- Set x = FD, y = BW, and a = 1.

- Set Inherit Dimension = No and Dimension = PowerSpectralFluxDensity.

- Click OK.

- Create a Coverage Definition.

- Set the Asset to be the Transmitter.

- Set the Grid to be the region of interest and the Point Granularity to be the appropriate value.

- Set the Grid Constraint Object to be the Receiver and select Use Actual Object on the Grid Points.

- Create a Figure of Merit as a child object of the Coverage Definition.

- Set the Figure of Merit to be of type Scalar Calculation.

- Set the Scalar Calculation to be SFD.

- Check Update Accesses computed outside of Coverage.

- For the Figure of Merit, select the Report and Graph Manager and run the Grid Stats report.

- Set the appropriate graphics levels and Legend for the Figure of Merit.