How to use Power Control to maintain a Constant Flux Density

Given a constant flux density, a transmitter model plugin script can be created to adjust the Transmit Power or EIRP accordingly.

Flux Density ( = EIRP/(4R2) 	(1)

EIRP = 4R2		

Where R = Range between transmitter and receiver

In MATLAB, the code is as follows to compute Power in Watts and convert to dBW:

r2 = range^2;

FD = 1e-7

pow = 10*log10(4*pi*r2*FD);

where

FD = Flux Density

pow = Transmit Power or EIRP

r2 = range squared

The computed power is assigned to the transmit power in the plugin script. A sample of this plugin in MATLAB is attached.

Sample output is shown here:

Time (EpSec)    EIRP (dBW)    Flux Density (dBW/m^2)

------------    ----------    ----------------------

1084.066        66.994                     -72.3

1144.000        65.593                     -72.3

1204.000        63.956                     -72.3

1264.000        62.033                     -72.3

1324.000        59.822                     -72.3

1384.000        57.614                     -72.3

1444.000        56.543                     -72.3

1504.000        57.647                     -72.3

1564.000        59.861                     -72.3

1624.000        62.069                     -72.3

1684.000        63.986                     -72.3

1744.000        65.619                     -72.3

1802.723        66.991                     -72.3