Systems Toolkit Tested Hardware Architecture and Virtualization Whitepaper

This whitepaper covers a tested successful bare metal configuration of a local, non-networked version of STK as well as 2 test cases virtualizing STK, both successful and unsuccessful.  Hardware configuration, operating systems, network configurations and virtualization scenarios will be covered.

For the virtual connections, the client connection software that was tested was using RealVNC VNC software (realvnc.com).  Microsoft’s Remote Desktop Protocol was not tested because of the known documented problems with OpenGL v.1.1 used by Microsoft.  Research indicated that Microsoft does not plan on upgrading their version of OpenGL and AGI indicates that OpenGL 1.x versions severely limit the capabilities of STK.

To test performance we used the Intro_STK_Space_Systems scenario.

Scenario 1 - Local, non-networked, bare metal server, STK 11.1.1, single user, no shared use of computer

Hardware & Operating System

Dell Precision T7910

28 Core Intel Xeon E5-2695 2.3 GHz CPUs

Windows Server 2012 R2

192GB RAM

NVIDIA Quadro K6000 Display Adapter (2), only one used

30” display

Windows 2012 R2

STK Performance was optimal.  Video frames per second (fps) optimized at 35 – 40fps.  Other user created scenarios managed up to 65 fps. There were no performance issues with STK.

System CPU and memory usage was minimal.  Both CPU and memory averaged between 2% – 3% that was attributable to STK (using Windows Task Manager and Resource Monitor).

Scenario 2 – Client connected to bare metal server with STK 11.1.1 via network, single user, no shared use of computer

Server Hardware & Operating System

Dell Precision T7910

28 Core Intel Xeon E5-2695 2.3 GHz CPUs

Windows Server 2012 R2

192GB RAM

NVIDIA Quadro K6000 Display Adapter (2), only one used

Windows Server 2012 R2

Client System

Generic HP PC with Intel i3 CPU 3GHz

4GB RAM

Windows 7 (64-bit)

Physical connection method to server was via a 1GB NIC on both the client and server on a 1GB VLAN.  Both the server and the client were on the same subnet, no network routing was incurred.

Client connection protocol used was Virtual Network Connection (TCP/Port 5900), using RealVNC software.

Performance was as good as being on the physical console of the bare metal system itself.  Frames per second were close, if not exact.  In addition, using the VNC software, native graphics on the server continued to render on the server and displayed to the remote connection as if using the server console.  Graphics card information displayed in the STK Help menu showed both of the NVIDIA Quadro K6000 graphics adapters.  Also, the OpenGL plugins were all there, 100+, so all OpenGL features were enabled.  In essence, there was no noticeable performance or video quality difference using a remote connection with VNC software, and STK reported the configuration as if I was on the server console itself.

There was also no increase in CPU or memory usage on the server or the client.

Scenario 3 – Virtualized STK 11.1.1 using Microsoft Hyper-V on the same bare metal system, single user, no shared use of computer

Server Hardware & Operating System

Dell Precision T7910

28 Core Intel Xeon E5-2695 2.3 GHz CPUs

Windows Server 2012 R2

192GB RAM

NVIDIA Quadro K6000 Display Adapter (2), only one used

Windows Server 2012 R2

Microsoft Hyper-V

RAM 12GB dedicated Video RAM,

RAM No dedicated system memory

RAM 98GB of shared system memory

CPU- used 1 NVIDIA GPU

Connected to Hyper-V virtual machine via the server’s desktop console.

Experienced numerous problems using this configuration with video and network.  The first problem is with the graphics card.  Hyper-V does not use the native graphics display adapter (NVIDIA Quadro K6000 in this case) for the graphics driver when connecting locally from the console, or remotely.  When configuring the VM, you can select the GPU on the video card as the processor.  However, Hyper-V does not indicate any vendor information on the GPU, only that there are GPUs on the server.  When connected to the VM and running STK, the graphics display adapter information showed, “Microsoft Basic Display Adapter, Renderer:  GDI Generic”.  As a result, the OpenGL version is 1.x and none of the modules that use OpenGL greater than v1.0 will work

The second problem was with networking.  When adding the Hyper-V service onto the machine and creating a VM, Hyper-V creates a virtual switch for networking.  By default, the virtual switch defaults to a speed of 10Mbps and the VM will use the virtual switch for its networking.  It may be configurable, but with the video rendering problem mentioned above we chose not to pursue attempting to get Hyper-V working.

We will be attempting test our VNC connection protocol across multiple network hops as soon as our network supports that protocol.  Test results will be provided when that occurs.